Specifications of Hot Air Gas Blower

The Hot Air Gas Blower system is used for variable temperature experiment with heating gas. The temperature range is from 25 to 1000 C.

Specifications:

| Quantity | 1 |
| --- | --- |
| Temperature range | 25~ 1000 C |
| Temperature stability | ± 0.5 C (not include below 250°C) |
| Temperature precision | ± 1 C |
| Pipe diameter | 8mm |
| Main power supply | 220V 1000W |
| Adjustable support stand for 8mm gas blower |  |
| Water safety device for cooling circuit |  |

Seller shall provide 1 year warranty from the date of inspection and acceptance passed by NSRRC.

Inspection & Acceptance Criteria:

- Please provide product catalogs.

- Please provide the factory acceptance test (FAT) report, including temperature range, temperature stability, and temperature precision.

- The pipe diameter, adjustable support, and cooling circuit water safety device will be verified via visual inspection; the main power supply will be inspected according to the specifications of product catalogs provided by your company.

- Upon receipt of the products, we will perform functional testing. The Hot Air Gas Blower will be validated by a heating process from room temperature to 1000 degrees Celsius to confirm its operational effectiveness and meet the specifications, including temperature stability and temperature precision.